CSE2414 REAL TIME PROGRAMMING  CAT-3 AND ASSIGNMENT

- Briefly explain co-operative multitasking and pre-emptive multitasking. Bring out the difference between these two context switching techniques. (8 marks)

- (ii) Briefly explain Advanced Configuration and Power Interface (ACPI). (5 marks)

- Using C language, write the code for the Earliest Deadline First (EDF) algorithm. (8 marks)

- How can the SWP instruction be used to implement atomic test-and-set in ARM architecture? (5 marks) (i)Define scheduling overhead. (3 marks)

- Determine the utilization of the given three processes. (10 marks)

Process Table:

| Process | Period (s) | Execution Time (s) |
| --- | --- | --- |
| P1 | 1.0 ×10-3 | 1.0 × 10-3 |
| P2 | 1.0 ×10-3 | 2.0 × 10-3 |
| P3 | 5.0 ×10-3 | 3.0 × 10-3 |

- Interpret and analyze the scheduling process by applying the Rate Monotonic Algorithm for the given set of processes. (15 marks)

Process Table:

| Process | Execution Time (units) | Period (units) |
| --- | --- | --- |
| P1 | 2 | 5 |
| P2 | 3 | 7 |
| P3 | 4 | 13 |

  - For the given set of periodic processes shown in the figure below, all of which share the same deadline of 18:

| Process | CPU Time (units) |
| --- | --- |
| P1 | 4 |
| P2 | 2 |
| l,P3 | 4 |

  - Schedule the processes for the given arrival times using standard Rate Monotonic Scheduling (no data dependencies). (7 marks)

  - Schedule the processes considering data dependencies. By how much is the CPU utilization reduced? (8 marks)

- Create the scheduling process by employing the Earliest Deadline First (EDF) algorithm for the given set of processes. (15 marks)

Process Table:

| Process | Execution Time (units) | Period (units) |
| --- | --- | --- |
| P1 | 2 | 6 |
| P2 | 2 | 8 |
| P3 | 4 | 10 |

- With suitable examples, explain the Accelerator System Design process. (13 marks)

- Discuss in detail the Distributed Embedded Architecture with a suitable example. (13 marks)

- With a neat block diagram, briefly explain how the CPU cache causes problems for accelerators, and suggest a technique to overcome the problem. (8 marks) (ii) With an example, explain how single-threaded or multi-threaded control of an accelerator affects the speedup factor. (5 marks)

- Explain Network-Based Real-Time System Design with suitable diagrams. (13 marks)

- Point out how the Internet can be used by Real-Time Computing Systems. (13 marks)

- Illustrate the I²C bus for an embedded system with meaningful diagrams. (13 marks)

- (i) Outline the concepts of Accelerated System Design in embedded systems. (8 marks) (ii) State the advantages of Accelerated System Design. (5 marks)

- Write short notes on: (i) CAN bus protocol. (7 marks) (ii) Ethernet. (6 marks)

- Narrate the detailed description of: (i) Crossbar Network. (6 marks) (ii) Multistage Network. (7 marks)

- (i) Describe the network abstractions with the help of OSI model layers. (6 marks) (ii) Demonstrate Network-Based System Design with necessary diagrams. (13 marks) (iii) Mention the different types of network buses used in creating the distributed Real-Time computing platform. With neat sketches, state transition graph and control word formats, briefly explain how data transaction takes place using the I²C bus. (13 marks)

- (i) Describe the different arbitration schemes with diagrams. (10 marks) (ii) Summarize Message Passing Programming. (3 marks)